Machine Learning and Cognitive Signal Processing Approaches for Radar-Based Drone Surveillance

The rapid proliferation of low-cost unmanned aerial vehicles has necessitated the development of sophisticated radar surveillance systems capable of reliable detection and classification [1]. Advanced signal processing methodologies are essential to distinguish these targets from clutter, particularly given the dynamic flight profiles and small radar cross-sections characteristic of modern UAVs [2], [3]. To address these technical hurdles, current frameworks incorporate space-time adaptive processing to mitigate clutter in three-dimensional space while maintaining sensitivity to low-velocity targets [4, p. 24]. Furthermore, the integration of digital beamforming and phased array architectures allows for improved spatial resolution, which is vital for managing the increased computational complexity associated with high-altitude monitoring [5, p. 7227]. Beyond spatial resolution, recent algorithmic innovations such as Range-Doppler Integration while Steering offer superior signal-to-noise ratio enhancement, significantly increasing detection probabilities for targets with minimal cross-sections [6]. Moreover, implementing track-free Automatic Target Recognition technologies reduces reliance on legacy tracking chains, enabling real-time responsiveness and autonomous operation in complex surveillance environments [7]. These systems further leverage micro-Doppler signatures to improve target classification accuracy, providing critical insights into the physical characteristics of the detected platforms [8, p. 1]. Additionally, researchers are increasingly employing cognitive radar techniques to adaptively select pulse repetition frequencies and modulation schemes, effectively resolving range and Doppler ambiguities inherent in challenging cluttered environments [9, p. 19]. These signal processing pipelines must also contend with the high degree of variability in micro-Doppler signatures, which are sensitive to the specific material composition of the drone's rotors [10, p. 11]. To mitigate these classification errors, data-driven approaches leverage convolutional neural networks to extract discriminative features from augmented radar datasets [11]. Furthermore, integrating multi-sensor fusion techniques with these deep learning architectures has shown significant promise in enhancing overall system reliability by cross-referencing radar telemetry with optical and acoustic data streams [12]. Beyond traditional signal-to-noise ratio metrics, emerging methodologies now utilize Doppler signal-to-clutter ratio detectors to maintain sensitivity at extended ranges, thereby decreasing false negative rates in low-RCS scenarios [13]. Moreover, implementing iterative adaptive approaches for Doppler processing further improves resolution and target detectability, effectively overcoming the dwell time constraints typical of conventional ground-based surveillance architectures [14]. Additionally, the integration of deep learning architectures, such as Recurrent Neural Networks, facilitates the classification of mini-drones by analyzing micro-Doppler spectrograms, thereby allowing cognitive systems to autonomously optimize waveform diversity for improved accuracy. Furthermore, the deployment of multilayer classification procedures enables the fusion of diverse domains—including cepstrograms and range-time profiles—to validate target identification within complex operational environments [15, p. 920]. In this context, the categorization of targets based on specific rotor kinematics—such as distinguishing between fixed-wing puller blades and multi-rotor lifting blades—is achieved by quantifying statistical metrics like Doppler frequency difference and magnitude ratio [16]. By synthesizing multidimensional features such as cadence-velocity diagrams with conventional spectrograms, these deep learning fusion models achieve classification accuracies exceeding 97% [17]. Beyond these classification benchmarks, contemporary research has extended this capability by utilizing resonant electromagnetic tags attached to drone blades, which generate configuration-specific returns to overcome the limitations of conventional radar cross-section profiles [18], [19]. Additionally, shifting focus toward multistatic configurations enables the construction of individual models for each radar viewpoint, which has proven more effective than aggregating features into a single high-dimensional input space [20, p. 12]. This shift toward viewpoint-specific modeling acknowledges the limitations of conventional Convolutional Neural Networks, which often struggle to account for the spatial diversity inherent in complex radar scattering [21, p. 153]. To overcome these architectural constraints, researchers are increasingly adopting lightweight deep convolutional neural networks, such as DIAT-RadSATNet, which utilize optimized layer configurations to minimize parameter overhead while maintaining high classification precision across diverse small UAV types [22]. Furthermore, employing these architectures in X-band radar scenarios has demonstrated robustness against variations in blade pitch, yielding significant F1-score improvements even at moderate signal-to-noise ratios [23]. Moreover, the implementation of Gated Recurrent Unit layers within these frameworks captures critical temporal dependencies, further bolstering resilience against performance degradation in noisy, long-range detection environments [24]. Finally, the integration of transfer learning strategies allows these models to generalize effectively across varied environments without requiring prohibitively large training sets [25]. By leveraging pre-trained weights from expansive datasets, fine-tuned models like EfficientNetB3 can achieve validation accuracies exceeding 97% even when faced with limited target-specific radar samples [26]. This advancement is further complemented by multi-scale residual architectures that fuse time-domain, time-frequency, and range-Doppler features to achieve identification accuracies reaching 94% [27]. Furthermore, recent developments in end-to-end learning models have successfully bypassed the computational overhead associated with traditional image-based feature extraction by directly processing raw radio frequency signatures [28].

Table 1: Summary of survey

| Ref. No. | Author(s) | Journal & Year | Problem Addressed | Methodology Adopted | Research Gap for Proposed Title | Next Level of Improvement Required |
| --- | --- | --- | --- | --- | --- | --- |
| [1] | Coluccia et al. | Sensors, 2020 | Reliable drone detection in radar networks | Review of radar sensor networks and classification methods | Lack of intelligent adaptive frameworks | AI-driven cognitive radar networks |
| [2] | Seidaliyeva et al. | Sensors, 2023 | Challenges in UAV detection and classification | Survey of radar, RF, optical methods | Limited real-time autonomous operation | Multi-modal intelligent systems |
| [3] | Sayed et al. | Drones, 2024 | Detection of low-RCS UAVs | FMCW radar with ML techniques | High false alarms in clutter | Cognitive waveform optimization |
| [4] | Bauw | Thesis, 2023 | Low-resolution target discrimination | One-class classification with limited supervision | Limited labeled datasets | Self-supervised learning models |
| [5] | Milias et al. | IEEE TITS, 2023 | Navigation and surveillance complexity | UAS-borne radar with digital beamforming | Computational overhead | Edge-AI-based beamforming |
| [6] | Sayed et al. | IEEE Sensors Journal, 2024 | Weak signal detection | RDIwS beamforming and Range-Doppler integration | Static processing parameters | Adaptive beam steering |
| [7] | Gong et al. | IRS Conference, 2025 | Real-time drone detection challenges | Automatic Target Recognition (ATR) | Dependence on tracking chains | End-to-end deep ATR systems |
| [8] | Gong et al. | arXiv, 2023 | Drone classification | Micro-Doppler based ATR | Sensitive to noise and clutter | Robust deep learning frameworks |
| [9] | Khawaja et al. | arXiv, 2024 | Detection and tracking limitations | Radar and communication system survey | Lack of cognitive adaptation | Reinforcement learning-based radar |
| [10] | Wang et al. | IEEE AES Magazine, 2021 | Counter-UAS challenges | Review of detection and tracking systems | Limited robustness to dynamic environments | Multi-domain fusion methods |
| [11] | Patil | ISJEM, 2025 | Small UAV detection | CNN using micro-Doppler signatures | Large training data requirement | Transfer learning and self-supervision |
| [12] | Semenyuk et al. | International Journal of Critical Infrastructure Protection, 2025 | Reliability of UAV detection | AI-based multi-sensor fusion | Sensor synchronization issues | Distributed sensor fusion networks |
| [13] | Gong et al. | Drones, 2023 | Detection at low SNR | Doppler Signal-to-Clutter Ratio detector | Performance degradation at long ranges | Adaptive clutter suppression |
| [14] | Sun et al. | IEEE TAES, 2019 | Poor Doppler resolution | Iterative Doppler processing | Long dwell time requirement | Fast iterative algorithms |
| [15] | Patel et al. | IET Radar Sonar & Navigation, 2018 | Small UAV classification | Micro-Doppler and RCS characterization | Limited feature diversity | Multi-feature deep fusion |
| [16] | Yan et al. | Drones, 2023 | Drone-type recognition | Rotor micro-Doppler analysis | Material-dependent signatures | Physics-informed learning models |
| [17] | Chen et al. | IEEE GRSL, 2024 | Classification accuracy limitations | Fusion of multidimensional micro-Doppler images | High computational complexity | Lightweight fusion networks |
| [18] | Yerushalimov et al. | arXiv, 2026 | Similar RCS signatures | μDopplerTag with CNN | Requires cooperative tags | Passive tag-free recognition |
| [20] | Gong et al. | arXiv, 2023 | ATR limitations in ground radar | Track-free ATR framework | Limited viewpoint information | Multistatic cognitive radar |
| [21] | Potter et al. | IEEE TAES, 2024 | Spatial diversity issue | Bayesian fusion in multistatic radar | High-dimensional feature space | Distributed Bayesian learning |
| [22] | Kumawat et al. | IEEE TIM, 2022 | Heavy deep models | DIAT-RadSATNet lightweight DCNN | Reduced temporal information | CNN-RNN hybrid models |
| [23] | Raval et al. | Drones, 2021 | UAV classification | CNN using X-band radar spectrograms | Sensitivity to noise | Attention-based CNN models |
| [24] | Agrebi et al. | RadarConf, 2025 | Noise effects on classification | GRU-based neural networks | Limited long-range performance | Transformer architectures |
| [25] | Kumawat et al. | IEEE GRSL, 2021 | Small dataset problem | CNN using micro-Doppler signatures | Poor generalization | Transfer learning and domain adaptation |
| [26] | Kaushal | IEEE EAIC, 2025 | Limited training samples | EfficientNetB3 transfer learning | Environment dependency | Self-supervised pretraining |
| [27] | Zhou et al. | IEEE GRSL, 2025 | Feature extraction limitations | Multiscale decision fusion CNN | High computational cost | Efficient multi-scale transformers |
| [28] | Alam et al. | Sensors, 2023 | Feature engineering complexity | End-to-end raw RF deep learning | Limited interpretability | Explainable AI for RF radar systems |

References:

- A. Coluccia, G. Parisi, and A. Fascista, “Detection and Classification of Multirotor Drones in Radar Sensor Networks: A Review,” Sensors, vol. 20, no. 15, pp. 4172–4172, Jul. 2020, doi: 10.3390/s20154172.

- U. Seidaliyeva, L. Ilipbayeva, K. Taissariyeva, Н. Смайлов, and E. T. Matson, “Advances and Challenges in Drone Detection and Classification Techniques: A State-of-the-Art Review,” Sensors, vol. 24, no. 1, pp. 125–125, Dec. 2023, doi: 10.3390/s24010125.

- A. N. Sayed, O. M. Ramahi, and G. Shaker, “Frequency-Modulated Continuous-Wave Radar Perspectives on Unmanned Aerial Vehicle Detection and Classification: A Primer for Researchers with Comprehensive Machine Learning Review and Emphasis on Full-Wave Electromagnetic Computer-Aided Design Tools,” Drones, vol. 8, no. 8, pp. 370–370, Aug. 2024, doi: 10.3390/drones8080370.

- M. Bauw, “One-class classification for low resolution targets discrimination with limited supervision in pulse Doppler radars,” Jan. 18, 2023. Accessed: May 2026. [Online]. Available: http://www.theses.fr/2023UPSLM005/document

- C. Milias et al., “UAS-Borne Radar for Autonomous Navigation and Surveillance Applications,” Huddersfield Research Portal (University of Huddersfield), vol. 24, no. 7, pp. 7215–7229, Mar. 2023, doi: 10.1109/tits.2023.3254582.

- A. N. Sayed, O. M. Ramahi, and G. Shaker, “RDIwS: An Efficient Beamforming-Based Method for UAV Detection and Classification,” IEEE Sensors Journal, vol. 24, no. 9, pp. 15230–15240, Mar. 2024, doi: 10.1109/jsen.2024.3375862.

- J. Gong, J. Yan, and D. Li, “Radar Challenges and Solutions for Drone Detection,” pp. 1–8, May 2025, doi: 10.23919/irs64527.2025.11046158.

- J. Gong, J. Yan, D. Kong, and D. Li, “Introduction to Drone Detection Radar with Emphasis on Automatic Target Recognition (ATR) technology,” Jul. 19, 2023, Cornell University. doi: 10.48550/arxiv.2307.10326.

- W. Khawaja, M. Ezuma, V. Semkin, F. Erden, Ö. Özdemir, and İ. Güvenç, “A Survey on Detection, Classification, and Tracking of UAVs using Radar and Communications Systems,” Feb. 08, 2024, Cornell University. doi: 10.48550/arxiv.2402.05909.

- J. Wang, Y. Liu, and H. Song, “Counter-Unmanned Aircraft System(s) (C-UAS): State of the Art, Challenges, and Future Trends,” arXiv (Cornell University), vol. 36, no. 3, pp. 4–29, Mar. 2021, doi: 10.1109/maes.2020.3015537.

- S. Patil, “A Data-Driven Approach to Small UAV Detection using Micro-Doppler Signatures and Deep Convolutional Architecture,” International Scientific Journal of Engineering and Management, vol. 4, no. 5, pp. 1–9, May 2025, doi: 10.55041/isjem03688.

- V. Semenyuk, I. Kurmashev, A. Lupidi, D. Alyoshin, L. Kurmasheva, and A. Cantelli‐Forti, “Advances in UAV detection: integrating multi-sensor systems and AI for enhanced accuracy and efficiency,” International Journal of Critical Infrastructure Protection, vol. 49, pp. 100744–100744, Feb. 2025, doi: 10.1016/j.ijcip.2025.100744.

- J. Gong, J. Yan, H. Hu, D. Kong, and D. Li, “Improved Radar Detection of Small Drones Using Doppler Signal-to-Clutter Ratio (DSCR) Detector,” Drones, vol. 7, no. 5, pp. 316–316, May 2023, doi: 10.3390/drones7050316.

- H. Sun, B. Oh, X. Guo, and Z. Lin, “Improving the Doppler Resolution of Ground-Based Surveillance Radar for Drone Detection,” IEEE Transactions on Aerospace and Electronic Systems, vol. 55, no. 6, pp. 3667–3673, Jan. 2019, doi: 10.1109/taes.2019.2895585.

- J. S. Patel, F. Fioranelli, and D. Anderson, “Review of radar classification and RCS characterisation techniques for small UAVs or drones,” IET Radar Sonar & Navigation, vol. 12, no. 9, pp. 911–919, Jul. 2018, doi: 10.1049/iet-rsn.2018.0020.

- J. Yan, H. Hu, J. Gong, D. Kong, and D. Li, “Exploring Radar Micro-Doppler Signatures for Recognition of Drone Types,” Drones, vol. 7, no. 4, pp. 280–280, Apr. 2023, doi: 10.3390/drones7040280.

- X. Chen, C. Ma, C. Zhao, and Y. Luo, “UAV Classification Based on Deep Learning Fusion of Multidimensional UAV Micro-Doppler Image Features,” IEEE Geoscience and Remote Sensing Letters, vol. 21, pp. 1–5, Jan. 2024, doi: 10.1109/lgrs.2024.3371171.

- O. Yerushalimov, D. Vovchuk, A. Glam, and P. Ginzburg, “μDopplerTag: CNN-Based Drone Recognition via Cooperative Micro-Doppler Tagging,” Jan. 12, 2026, Cornell University. doi: 10.48550/arxiv.2601.08042.

- O. Yerushalimov, D. Vovchuk, A. Glam, and P. Ginzburg, “μDopplerTag: CNN-Based Drone Recognition via Cooperative Micro-Doppler Tagging,” ArXiv.org, Jan. 2026, Accessed: Feb. 2026. [Online]. Available: http://arxiv.org/abs/2601.08042

- J. Gong, J. Yan, D. Kong, and D. Li, “An introduction to radar Automatic Target Recognition (ATR) technology in ground-based radar systems,” Oct. 23, 2023, Cornell University. doi: 10.48550/arxiv.2310.14769.

- M. Potter, M. Akçakaya, M. Necsoiu, G. Schirner, D. Erdoğmuş, and T. Imbiriba, “Multistatic-Radar RCS-Signature Recognition of Aerial Vehicles: A Bayesian Fusion Approach,” Aug. 19, 2024, Cornell University. doi: 10.1109/taes.2024.3445323.

- H. C. Kumawat, M. Chakraborty, and A. A. B. Raj, “DIAT-RadSATNet—A Novel Lightweight DCNN Architecture for Micro-Doppler-Based Small Unmanned Aerial Vehicle (SUAV) Targets’ Detection and Classification,” IEEE Transactions on Instrumentation and Measurement, vol. 71, pp. 1–11, Jan. 2022, doi: 10.1109/tim.2022.3188050.

- D. Raval, E. Hunter, S. Hudson, A. Damini, and B. Balaji, “Convolutional Neural Networks for Classification of Drones Using Radars,” Drones, vol. 5, no. 4, pp. 149–149, Dec. 2021, doi: 10.3390/drones5040149.

- A. Agrebi, X. Neyt, and F. Horlin, “Tackling the Effect of Noise in the Context of a Neural Network-Based Unmanned Aerial Vehicles (UAVs) Classification from Radar Data,” pp. 1–7, Oct. 2025, doi: 10.1109/radarconf2559087.2025.11205022.

- H. C. Kumawat, M. Chakraborty, A. A. B. Raj, and S. Dhavale, “DIAT-μSAT: Small Aerial Targets’ Micro-Doppler Signatures and Their Classification Using CNN,” IEEE Geoscience and Remote Sensing Letters, vol. 19, pp. 1–5, Aug. 2021, doi: 10.1109/lgrs.2021.3102039.

- D. Kaushal, “Radar-Based UAV Classification: A Micro-Doppler and Deep Learning Approach,” pp. 1–6, Jun. 2025, doi: 10.1109/eaic66483.2025.11101447.

- H. Zhou, Y. Chen, A. Gong, Y. Zhu, C. Mo, and W. Xie, “UAV Identification via Multiscale Decision Fusion CNN Utilizing Micro-Doppler Features,” IEEE Geoscience and Remote Sensing Letters, vol. 22, pp. 1–5, Jan. 2025, doi: 10.1109/lgrs.2025.3574642.

- S. S. Alam et al., “RF-Enabled Deep-Learning-Assisted Drone Detection and Identification: An End-to-End Approach,” Sensors, vol. 23, no. 9, pp. 4202–4202, Apr. 2023, doi: 10.3390/s23094202.